DCC-MI-DC-DOC.F-0001_04

| 00 | 30 Dec.2021 |  | MM |  |  |  |  |  | For approval |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rev | Date |  | Originator |  | Signature | ABV Approval | Signature |  | Revision Description |  |  |  |  |
| Owner: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECURITY FORCES MEDICAL CENTER PROJECT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consultant: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Subcontractor: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Non-ABV Reference: |  |  |  |  |  |  |  | Scale: |  | Floor |  | Document Type: |  |
| Document Title: ABV PROJECTS OPERATING MANUAL Method Of Statement for the Steel Floor Bracket Installation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Document Number: MI; C0; MS; 00; 00. Proj; Orig.; Package; Building; Discipline; Doc Type; Sequence; Phase; Revision; Issue. |

Contents

- Contractor or Sub Contractor Details

- Scope/Summary of works

- Programme Duration of Activity

- Management Structure/Resources and Responsibilities

- Communication/Contact Details

- Method Statement

- Inspection and Test Plan (and Quality Records)

- Tools & Equipment

- Special Scaffold/Access Equipment

- First Aid and Emergency Arrangements (Including e.g., fall arrest, confined space rescue)

- Advise Any Permits to Work Required (e.g., Hot works; confined space; excavation; electrical)

- Environmental Considerations

- Waste Management

- Hazardous Substances – Attach material data sheets

- Any Special Storage Requirements

- Testing and Commissioning

- Supporting Documents

1.0 	Contractor/Sub contractor Details

The works will be undertaken by AMG in the presents of an ABV representative on site.

2.0 	Scope of Work

Steel floor bracket installation with 20mm anchor (Mechanical or Chemical) bolt.

3.0 	Programme Duration of Activity

As and when required.

4.0	Management Structure/Resources and Responsibilities

The overall works will be managed by AMG under ABV supervision. Minimum PPE of safety helmet, high visibility vest, and safety footwear will be managed.

5.0 	ABV & AMG Communication/Contact Details

Melter Rendon 	(ABV Construction Manager)			0556149668

MohammadJamal	(ABV QA/QC Supervisor) 			0536179335

Sahul Hameed           (AMG Project Manager)		          0598763151

Afsal Ninakutty 	(AMG Construction Manager)		          0500681299

Abdul Mubeen           (AMG QA/QC Engineer)                              0536475263

Jinto Jose 		(AMG HSE)					0594089763

6.0 	METHOD STATEMENT

6.1	Set the survey markings of the steel floor bracket location based on the approved layout drawing and prepare the concrete surface.

6.2 Create a drill hole reaching the installation depth specified in the data sheet of 20mm diam. - Fischer Bolt anchor FAZ II or Fischer Injection System FIS EM Plus (or any equivalent approved products) using hammer drill.

6.3 Clean the drill holes using blow out or vacuum cleaner.

6.4 Installation of Anchor Bolt

For using the mechanical anchor bolt (20mm diam. - Fischer Bolt anchor FAZ II or any equivalent product): Insert the anchor into the drill hole and drive-in using manual hammer and initially set.

For using the chemical anchor bolt (20mm Fischer Injection System FIS EM Plus or any equivalent product): Screw the tube into the cartridge and position the assembly in the application gun. Insert the tube to the bottom of the drill hole and apply the mortar to ½ and ¾ of its depth. Introduce the stud to be installed by screwing it lightly down to the installation depth value manually; ensuring the mortar covers the stud thread. The mortar must seep from the top of the drill hole to ensure it is completely full and there are no gaps between the stud and drill hole. Allow the mortar to cure as per manufacturer recommendation.

6.5. Install the EPDM Separator

6.6 Install the steel floor bracket and align accordingly.

6.7 Apply the torque of the anchor bolt specified in the data sheet of 20mm diam. - Fischer Bolt anchor FAZ II or Fischer Injection System FIS EM Plus (or any equivalent approved products) using torque wrench.

7.0 	Inspection:

* Hold Point

* An engineer’s inspection will be submitted before work commences.

* An engineer’s inspection of the completed bracket installation will be requested.

Records:

* Inspection Request documentation

8.0 	Tools & Equipment

Drilling or Coring Machine

Extension Wire

Chemical

Water

Hammer

Spanner

Torque wrench

9.0 	Special Scaffold/Access Equipment

None

10.0 	First Aid and Emergency Arrangements (Including e.g., fall arrest, confined space rescue)

The site has a fully operational clinic and ambulance manned by qualified personnel

11.0	Advise Any Permits to Work Required (e.g., Hot works; confined space; excavation; electrical)

Hot work permits will be furnished, and a Toolbox talk will be conducted by AMG / ABV HSE department.

12.0 	Environmental Considerations

None

13.0 	Waste Management

Removed concrete or waste will be dumped in appropriate bins.

14.0 	Hazardous Substances – Attach material data sheets

None

15.0 	Any Special Storage Requirements

None

16.0 	Testing and Commissioning

Pull Out Test

17.0 	Supporting Documents:

- Risk Assessments (including personal protective equipment required)

- Structural calculation

- Technical Data (Anchor)